# SAFETY TRAINING EXAM

| Instructor Name: | Name of Student: |
| --- | --- |
| Completed Name: | Date of Forklift Safety Training: |

- How often does a forklift operator need to go through forklift PIT safety evaluation?

- Annually

- Every 3 years

- Certification is not needed once certified

- Every 2 years

- When does refresher training need to be conducted?

- If you or someone observes the operator using the forklift in an unsafe manner

- If the operator has an accident or near miss

- If the operator is assigned to drive a different type of forklift

- If a workplace change affects safety

- If the operator fails an evaluation

- All of the above

- Forklift safety certification shall include the following:

- Name of the operator

- Date of the training

- Date of the evaluation

- Identify the person(s) preforming the training or evaluation

- All of the above

- True or False. You must be considered a competent and knowledgeable in a specific forklift class and type in order to operate that forklift?

- _________________________________________________________________________________

- True or False. A scissor lift is considered a powered industrial truck and requires safety training certification as defined in the OSHA Powered Industrial Truck standard?

- ________________________________________________________________________________

- True or False. There is approximately 1 workplace fatality every 3 days involving a forklift?

- ________________________________________________________________________________

- True or False. It’s ok to use internal combustion engines in environments with poor ventilation.

- ________________________________________________________________________________

- What class are electric motor narrow aisle trucks part of?

- Class I

- Class IV

- Class III

- Class II

- True or False.  Manual pallet jack operators must go through a forklift certification course?

- ________________________________________________________________________________

- How often should an operator conduct a forklift inspection?

- Before the start of each shift on a daily basis?

- Every other day

- Once a week

- Annually

- True or False. It’s ok to operate a forklift that is leaking a small amount of hydraulic fluid?

- ________________________________________________________________________________

- True or False. We should visually inspect the mast chains to ensure there are no kinks, breaks, or lack
of lubrication?

- ________________________________________________________________________________

- Which of the following forklift systems should we check to make sure they are functioning properly?

- Seatbelts

- Lights

- Mirrors

- Backup Alarms

- Horns

- All of the above

- True or False. It’s important to check the forklift blades for wear, surface cracks and straightness?

________________________________________________________________________________

- Surface cracks often occur in what section of the forks?

- The tip

- The heel, at the bend

- The shank

- The blade

- True or False. Too much or too little play on the steering wheel can affect control of your forklift?

- ________________________________________________________________________________

- True or False. The forklift operator is responsible for making all necessary repairs on their lift.
 ________________________________________________________________________________

- True or False. You should always use a three point contact when entering or exiting a forklift.

- ________________________________________________________________________________

- True or False. It’s ok not to use your seatbelt if only operating your forklift in short distances?

- ________________________________________________________________________________

- In the event your forklift begins to tip-over you should:

- Never jump

- Plant your feet on the vehicles floor

- Grasp the steering wheel

- Lean forward and away from the impact

- All of the above

- What are some of the challenges that make it difficult to operate a forklift?

- Variable weight

- Lack of suspension

- Rear-wheel steering

- Long stopping distance

- All of the above

- When moving a load with a forklift you should:

- Tilt the mast back while traveling

- Carry the load as low as possible

- Drive slowly

- Stop at blind intersections  and sound your horn

- Look before backing up

- All of the above

- True or False. If you have a back up alarm on your forklift you don’t need to look before backing up?
________________________________________________________________________________

- True or False. It’s ok to pass other forklifts at intersections. ________________________________________________________________________________

- True or False.  You should never stick your arms or legs outside of your forklift. ________________________________________________________________________________

- If your driving uphill with a load, and your load restricts your view, you should:

- Be aware of your surroundings

- Lean outside of the lift to get a better view

- Drive in reverse with large loads

- Drive forward slowly, lead with your forks and make use of a spotter

- True or False. If you’re driving uphill without a load, lead with the rear of a forklift, drive in reverse with forks pointed downgrade. ________________________________________________________________________________

- True or False. If you’re driving downhill without a load, lead with forks pointed downgrade and
drive forward.

- ________________________________________________________________________________

- True or False. If you’re driving downhill with a load, lead with the rear of the forklift and drive in reverse.

- ________________________________________________________________________________

- True or False. OSHA considers a forklift unattended once it’s not in the view of the operator. ________________________________________________________________________________

- True or False. A forklift suspension is similar to a car. ________________________________________________________________________________

- True or False. Liquid Petroleum gas is extremely cold and can cause freeze burn if you don’t wear the appropriate gloves when refueling.

- ________________________________________________________________________________

- True or False. Forklift batteries should be charged in a designated area with adequate ventilation, fire extinguisher and eyewash station.

- ________________________________________________________________________________

- True or False. When charging and servicing batteries - pour acid into water, never water into acid. ________________________________________________________________________________

- True or False. When lifting several items, you should place the heaviest items towards the back of the forks in order to maintain stability.

- ________________________________________________________________________________

- You should do the following when loading in trailers and trucks.

- Chock the wheels

- Check the flooring inside the trailer for damage and capacity to support your forklift

- Inspect the dock board to make sure it’s secure and stable

- Control the keys

- All of the above

- True or False. Never allow anyone to walk, stand or pass under a raised load. ________________________________________________________________________________

- True or False. Raising a load on a counterbalance forklift, moves the combined center of gravity closer to the edge of stability pyramid. ________________________________________________________________________________

- What are examples of forklift tip over risk factors?

- Raising the load

- Adding weight

- Driving fast

- Stopping or turning

- All of the above

- True or False. It’s important not to combine multiple tip over risk factors. ________________________________________________________________________________

- True or False. It’s important to determine your load weight in order to prevent your forklift from
tipping over.

- ________________________________________________________________________________

- True or False. You should always account for the weight of any attachments or accessories used on
a forklift. ________________________________________________________________________________

- Where can you obtain the maximum load capacity of your forklift?

- Capacity plate on the forklift

- Google it

- From your supervisor

- On a different similar forklift

- True or False. The maximum load capacity of a forklift is reduced when you are lifting an object with a load center that is greater than the standard load center.  ________________________________________________________________________________

- True or False. Never lift someone that is standing on the forks or a pallet. ________________________________________________________________________________

- True or False. It’s ok to lift someone on a pallet if you’re not lifting them over 6 feet. ________________________________________________________________________________

- Forklift work platforms must be:

- Designed and certified by a manufacturer or professional engineer

- Securely attached to the forks

- Equipped with: guardrails, fall protection and signs showing capacity

- Monitored at all times by the forklift operator

- All of the above

- When entering or leaving a building with blind intersections you should:

- Stop and drive out

- Drive right out

- Stop, sound horn, and go out slowly

- Slow down

- True or False. Bent or damaged loading pallets must never be used.

- ________________________________________________________________________________

- True or False. Forklifts should be operated at full speed in order to get the work done faster.

- ________________________________________________________________________________

icwgroup.com

ICW Group is the marketing name for ICW Group Holdings, Inc. For a list of all ICW Group Holdings, Inc. subsidiaries, please visit our website www.icwgroup.com. 
Not all products and coverages are available in all states.